MQTT Packet Specification

AC Control — Cloud ↔ Firmware Interface

Unimation BLE Mesh   |   Version 1.0   |   Wed Aug 19 2026

1. Overview

This document defines all MQTT packets related to AC control between the cloud and the Unimation BLE Mesh firmware. All packets are JSON objects published to or received from the MQTT broker at 195.250.30.213:1883.

| Packet | ID | Direction | Topic | Trigger |
| --- | --- | --- | --- | --- |
| GWY AC Control Command | 3 | Cloud → Gateway | {GwySerNo}/command | App sends AC command to gateway |
| NODE AC Control Command | 103 | Cloud → Gateway → Node | {GwySerNo}/command | App sends AC command to node |
| GWY AC Control ACK | 13 | Gateway → Cloud | {GwySerNo}/message | Gateway confirms IR transmitted |
| NODE AC Control ACK | 113 | Node → Gateway → Cloud | {GwySerNo}/message | Node confirms IR transmitted |
| GWY Manual AC Control ACK | 4 | Gateway → Cloud | {GwySerNo}/message | Physical remote used on gateway |
| NODE Manual AC Control ACK | 104 | Node → Gateway → Cloud | {GwySerNo}/message | Physical remote used on node |

ℹ  All packets are published to {GwySerNo}/command (cloud→device) or {GwySerNo}/message (device→cloud). Replace {GwySerNo} with the actual gateway serial number e.g. GWY00013.

2. Commands from Cloud to Device

These packets are sent by the cloud to control the AC. The firmware validates every field before executing the command. An ACK is always returned regardless of success or failure.

2.1 GWY AC Control Command (JsonPacketID: 3) — Cloud → Gateway

Send this packet to control the AC connected to the gateway's own IR transmitter.

Fields

| Field | Type | Description |
| --- | --- | --- |
| JsonPacketID | int | 3 for GWY, 103 for NODE |
| MsgSeqNo | int | Sequence number 0–65535. Increment per command. Echoed back in ACK for matching. |
| Power | int | 1 = On, 0 = Off |
| Mode | string | Cool ¦ Hot ¦ Auto ¦ Dry ¦ Fan |
| FanSpeed | int | 0 = Auto, 1 = Low, 2 = Medium, 3 = High, 4 = Very High, 5 = Max |
| Temperature | int | Target temperature in °C. Range: 18–32 |
| SwingH | int | Horizontal swing. 1 = On, 0 = Off |
| SwingV | int | Vertical swing. 1 = On, 0 = Off |
| OnTimer | int | On timer in hours. 0 = disabled. Range: 0–12 |
| OffTimer | int | Off timer in hours. 0 = disabled. Range: 0–12 |
| Locking | int | Temperature locking. 1 = enabled, 0 = disabled |
| TempLockUpLimit | int | Upper lock limit in °C. Range: 18–32. Must be ≥ TempLockLowLimit |
| TempLockLowLimit | int | Lower lock limit in °C. Range: 18–32. Must be ≤ TempLockUpLimit |

Sample Packet

{

"JsonPacketID": 3,

"MsgSeqNo": 10,

"Power": 1,

"Mode": "Cool",

"FanSpeed": 3,

"Temperature": 24,

"SwingH": 0,

"SwingV": 0,

"OnTimer": 0,

"OffTimer": 0,

"Locking": 1,

"TempLockUpLimit": 28,

"TempLockLowLimit": 22

}

2.2 NODE AC Control Command (JsonPacketID: 103) — Cloud → Gateway → Node

Send this packet to control the AC connected to a specific BLE Mesh node. The gateway forwards this command to the node identified by ElementAddr over BLE Mesh.

Fields

| Field | Type | Description |
| --- | --- | --- |
| ElementAddr | int | BLE Mesh unicast address of the target node (assigned during provisioning) |
| JsonPacketID | int | 3 for GWY, 103 for NODE |
| MsgSeqNo | int | Sequence number 0–65535. Increment per command. Echoed back in ACK for matching. |
| Power | int | 1 = On, 0 = Off |
| Mode | string | Cool ¦ Hot ¦ Auto ¦ Dry ¦ Fan |
| FanSpeed | int | 0 = Auto, 1 = Low, 2 = Medium, 3 = High, 4 = Very High, 5 = Max |
| Temperature | int | Target temperature in °C. Range: 18–32 |
| SwingH | int | Horizontal swing. 1 = On, 0 = Off |
| SwingV | int | Vertical swing. 1 = On, 0 = Off |
| OnTimer | int | On timer in hours. 0 = disabled. Range: 0–12 |
| OffTimer | int | Off timer in hours. 0 = disabled. Range: 0–12 |
| Locking | int | Temperature locking. 1 = enabled, 0 = disabled |
| TempLockUpLimit | int | Upper lock limit in °C. Range: 18–32. Must be ≥ TempLockLowLimit |
| TempLockLowLimit | int | Lower lock limit in °C. Range: 18–32. Must be ≤ TempLockUpLimit |

ℹ  ElementAddr is the BLE Mesh unicast address assigned to the node during provisioning. It is included in all node provisioning ACK packets.

Sample Packet

{

"JsonPacketID": 103,

"MsgSeqNo": 10,

"ElementAddr": 5,

"Power": 1,

"Mode": "Cool",

"FanSpeed": 3,

"Temperature": 24,

"SwingH": 0,

"SwingV": 0,

"OnTimer": 0,

"OffTimer": 0,

"Locking": 1,

"TempLockUpLimit": 28,

"TempLockLowLimit": 22

}

3. AC Control ACK — Device to Cloud

These packets are sent by the firmware after executing an AC control command from the cloud. They confirm the result and include the full AC state at the time of IR transmission.

3.1 GWY AC Control ACK (JsonPacketID: 13) — Gateway → Cloud

Sent by the gateway after it executes a GWY AC Control Command (ID:3).

Fields

| Field | Type | Description |
| --- | --- | --- |
| JsonPacketID | int | Always 13 for GWY AC Control ACK |
| MsgSeqNo | int | Echoed from the original command packet |
| GwySerNo | string | Gateway serial number e.g. GWY00013 |
| ErrorCode | int | 0 = SUCCESS. See error code table below |
| ErrorMsg | string | Human readable error description |
| currACState | object | Full AC state at time of transmission. See currACState fields table |

currACState Object Fields

| Field | Type | Description |
| --- | --- | --- |
| Power | int | AC power state at time of transmission. 1 = On, 0 = Off |
| Mode | string | AC mode. Cool ¦ Hot ¦ Auto ¦ Dry ¦ Fan |
| FanSpeed | int | Fan speed level |
| Temperature | int | Set temperature in °C |
| AmbientTemperature(Digital) | int | I2C digital sensor reading. Note: calibration in progress — do not use as °C yet |
| AmbientTemperature(Analog) | int | NTC analog sensor reading in °C — use this as the reliable ambient reading |
| SwingH | int | Horizontal swing state |
| SwingV | int | Vertical swing state |
| OnTimer | int | On timer value in hours |
| OffTimer | int | Off timer value in hours |
| Locking | int | Locking state. 1 = enabled, 0 = disabled |
| TempLockUpLimit | int | Upper lock limit in °C |
| TempLockLowLimit | int | Lower lock limit in °C |

Sample Packet

{

"JsonPacketID": 13,

"MsgSeqNo": 10,

"GwySerNo": "GWY00013",

"ErrorCode": 0,

"ErrorMsg": "SUCCESS",

"currACState": {

"Power": 1,

"Mode": "Cool",

"FanSpeed": 3,

"Temperature": 24,

"AmbientTemperature(Digital)": 109,

"AmbientTemperature(Analog)": 28,

"SwingH": 0,

"SwingV": 0,

"OnTimer": 0,

"OffTimer": 0,

"Locking": 1,

"TempLockUpLimit": 28,

"TempLockLowLimit": 22

}

}

3.2 NODE AC Control ACK (JsonPacketID: 113) — Node → Gateway → Cloud

Sent by the gateway after the node confirms IR transmission over BLE Mesh. The ambient temperature values in currACState are read from the node's onboard sensors.

Fields

| Field | Type | Description |
| --- | --- | --- |
| JsonPacketID | int | Always 113 for NODE AC Control ACK |
| MsgSeqNo | int | Echoed from the original command packet |
| GwySerNo | string | Gateway serial number e.g. GWY00013 |
| NodeSerNo | string | Node serial number e.g. N00012 |
| ElementAddr | int | BLE Mesh unicast address of the node |
| RSSI_dBm | int | BLE signal strength between gateway and node in dBm |
| ErrorCode | int | 0 = SUCCESS. See error code table below |
| ErrorMsg | string | Human readable error description |
| BleErrorCode | int | 0 = BLE Mesh transmission success. Non-zero = BLE error |
| currACState | object | Full AC state at time of transmission. See currACState fields table |

currACState Object Fields

| Field | Type | Description |
| --- | --- | --- |
| Power | int | AC power state at time of transmission. 1 = On, 0 = Off |
| Mode | string | AC mode. Cool ¦ Hot ¦ Auto ¦ Dry ¦ Fan |
| FanSpeed | int | Fan speed level |
| Temperature | int | Set temperature in °C |
| AmbientTemperature(Digital) | int | I2C digital sensor reading. Note: calibration in progress — do not use as °C yet |
| AmbientTemperature(Analog) | int | NTC analog sensor reading in °C — use this as the reliable ambient reading |
| SwingH | int | Horizontal swing state |
| SwingV | int | Vertical swing state |
| OnTimer | int | On timer value in hours |
| OffTimer | int | Off timer value in hours |
| Locking | int | Locking state. 1 = enabled, 0 = disabled |
| TempLockUpLimit | int | Upper lock limit in °C |
| TempLockLowLimit | int | Lower lock limit in °C |

Sample Packet

{

"JsonPacketID": 113,

"MsgSeqNo": 10,

"GwySerNo": "GWY00013",

"NodeSerNo": "N00012",

"ElementAddr": 5,

"RSSI_dBm": -36,

"ErrorCode": 0,

"ErrorMsg": "SUCCESS",

"BleErrorCode": 0,

"currACState": {

"Power": 1,

"Mode": "Cool",

"FanSpeed": 3,

"Temperature": 24,

"AmbientTemperature(Digital)": 109,

"AmbientTemperature(Analog)": 28,

"SwingH": 0,

"SwingV": 0,

"OnTimer": 0,

"OffTimer": 0,

"Locking": 1,

"TempLockUpLimit": 28,

"TempLockLowLimit": 22

}

}

4. Manual AC Control ACK — Device to Cloud

These packets are sent automatically when a user presses the physical AC remote near the gateway or node. No cloud command triggers these — they are initiated by the firmware's IR receiver detecting a physical remote press.

ℹ  If the locking feature is active and the detected temperature is outside the locked range, the firmware overrides the AC back to the allowed range before sending this ACK. The currACState object reflects the final corrected state.

4.1 GWY Manual AC Control ACK (JsonPacketID: 4) — Gateway → Cloud

Sent when a physical remote is used near the gateway unit.

Fields

| Field | Type | Description |
| --- | --- | --- |
| JsonPacketID | int | Always 4 for GWY Manual AC Control ACK |
| GwySerNo | string | Gateway serial number |
| Power | int | Detected power state from physical remote press |
| DetectedTemperature | int | Temperature detected from physical remote IR signal |
| Temperature | int | Last set temperature from cloud command (for locking comparison) |
| FanSpeed | int | Detected fan speed from physical remote IR signal |
| Mode | string | Detected mode from physical remote IR signal |
| PowerError | int | 0 = Power detected successfully. Non-zero = could not parse power from IR |
| TemperatureError | int | 0 = Temperature detected successfully. Non-zero = could not parse temp from IR |
| FanSpeedError | int | 0 = Fan speed detected successfully. Non-zero = could not parse fan from IR |
| ModeError | int | 0 = Mode detected successfully. Non-zero = could not parse mode from IR |
| currACState | object | Full AC state after locking correction (if any). See currACState fields table |

currACState Object Fields

| Field | Type | Description |
| --- | --- | --- |
| Power | int | AC power state at time of transmission. 1 = On, 0 = Off |
| Mode | string | AC mode. Cool ¦ Hot ¦ Auto ¦ Dry ¦ Fan |
| FanSpeed | int | Fan speed level |
| Temperature | int | Set temperature in °C |
| AmbientTemperature(Digital) | int | I2C digital sensor reading. Note: calibration in progress — do not use as °C yet |
| AmbientTemperature(Analog) | int | NTC analog sensor reading in °C — use this as the reliable ambient reading |
| SwingH | int | Horizontal swing state |
| SwingV | int | Vertical swing state |
| OnTimer | int | On timer value in hours |
| OffTimer | int | Off timer value in hours |
| Locking | int | Locking state. 1 = enabled, 0 = disabled |
| TempLockUpLimit | int | Upper lock limit in °C |
| TempLockLowLimit | int | Lower lock limit in °C |

Sample Packet

{

"JsonPacketID": 4,

"GwySerNo": "GWY00013",

"Power": 1,

"DetectedTemperature": 24,

"Temperature": 24,

"FanSpeed": 3,

"Mode": "Cool",

"PowerError": 0,

"TemperatureError": 0,

"FanSpeedError": 0,

"ModeError": 0,

"currACState": {

"Power": 1,

"Mode": "Cool",

"FanSpeed": 3,

"Temperature": 24,

"AmbientTemperature(Digital)": 109,

"AmbientTemperature(Analog)": 28,

"SwingH": 0,

"SwingV": 0,

"OnTimer": 0,

"OffTimer": 0,

"Locking": 1,

"TempLockUpLimit": 28,

"TempLockLowLimit": 22

}

}

4.2 NODE Manual AC Control ACK (JsonPacketID: 104) — Node → Gateway → Cloud

Sent when a physical remote is used near a node unit. The gateway relays this to MQTT after receiving it over BLE Mesh.

Fields

| Field | Type | Description |
| --- | --- | --- |
| JsonPacketID | int | Always 104 for NODE Manual AC Control ACK |
| GwySerNo | string | Gateway serial number |
| NodeSerNo | string | Node serial number e.g. N00012 |
| ElementAddr | int | BLE Mesh unicast address of the node |
| RSSI_dBm | int | BLE signal strength between gateway and node in dBm |
| Power | int | Detected power state from physical remote press |
| DetectedTemperature | int | Temperature detected from physical remote IR signal |
| FanSpeed | int | Detected fan speed from physical remote IR signal |
| Mode | string | Detected mode from physical remote IR signal |
| PowerError | int | 0 = detected successfully. Non-zero = parse error |
| TemperatureError | int | 0 = detected successfully. Non-zero = parse error |
| FanSpeedError | int | 0 = detected successfully. Non-zero = parse error |
| ModeError | int | 0 = detected successfully. Non-zero = parse error |
| currACState | object | Full AC state including ambient temps and locking state. See currACState fields table |

currACState Object Fields

| Field | Type | Description |
| --- | --- | --- |
| Power | int | AC power state at time of transmission. 1 = On, 0 = Off |
| Mode | string | AC mode. Cool ¦ Hot ¦ Auto ¦ Dry ¦ Fan |
| FanSpeed | int | Fan speed level |
| Temperature | int | Set temperature in °C |
| AmbientTemperature(Digital) | int | I2C digital sensor reading. Note: calibration in progress — do not use as °C yet |
| AmbientTemperature(Analog) | int | NTC analog sensor reading in °C — use this as the reliable ambient reading |
| SwingH | int | Horizontal swing state |
| SwingV | int | Vertical swing state |
| OnTimer | int | On timer value in hours |
| OffTimer | int | Off timer value in hours |
| Locking | int | Locking state. 1 = enabled, 0 = disabled |
| TempLockUpLimit | int | Upper lock limit in °C |
| TempLockLowLimit | int | Lower lock limit in °C |

Sample Packet

{

"JsonPacketID": 104,

"GwySerNo": "GWY00013",

"NodeSerNo": "N00012",

"ElementAddr": 5,

"RSSI_dBm": -36,

"Power": 1,

"DetectedTemperature": 24,

"FanSpeed": 3,

"Mode": "Cool",

"PowerError": 0,

"TemperatureError": 0,

"FanSpeedError": 0,

"ModeError": 0,

"currACState": {

"Power": 1,

"Mode": "Cool",

"FanSpeed": 3,

"Temperature": 24,

"AmbientTemperature(Digital)": 109,

"AmbientTemperature(Analog)": 28,

"SwingH": 0,

"SwingV": 0,

"OnTimer": 0,

"OffTimer": 0,

"Locking": 1,

"TempLockUpLimit": 28,

"TempLockLowLimit": 22

}

}

5. Error Codes

The ErrorCode field in all ACK packets indicates the result of the operation. When ErrorCode is non-zero the command was not executed and currACState may contain stale values.

| Code | Name | Meaning |
| --- | --- | --- |
| 0 | SUCCESS | Command accepted and IR transmitted successfully |
| 1 | FAILURE | Generic failure |
| 9 | NODE_COMM_TIMEOUT | Node did not respond within BLE Mesh timeout |
| 13 | GWY_NOT_REG | Gateway not registered |
| 14 | GWY_ALREADY_REG | Gateway already registered |
| 15 | DEVICE_NOT_CONFIGURED | Device not yet configured with AC remote |
| 16 | MISSING_POWER | Power field missing from command |
| 17 | MISSING_TEMPERATURE | Temperature field missing from command |
| 18 | TEMPERATURE_EXCEEDING_RANGE | Temperature out of 18–32°C range |
| 23 | MISSING_MODE | Mode field missing from command |
| 40 | PUBLISH_PERIOD_EXCEEDING_RANGE | Publish period below 300s minimum |

6. Implementation Notes

1.  MsgSeqNo must be incremented by the cloud for each new command sent. The firmware echoes it back in the ACK so the cloud can match responses to requests. Valid range: 0–65535, wraps back to 0.

2.  All packets are published to the same topic — {GwySerNo}/message for device-to-cloud and {GwySerNo}/command for cloud-to-device. Use JsonPacketID to distinguish packet types.

3.  AmbientTemperature(Digital) values are currently under calibration. Use AmbientTemperature(Analog) as the reliable ambient temperature reading until further notice.

4.  For NODE packets, RSSI_dBm is the BLE Mesh signal strength between gateway and node measured at the gateway when the node's BLE ACK was received.

5.  BleErrorCode 0 means BLE Mesh transmission succeeded. Non-zero means the gateway could not deliver the command to the node or did not receive a response within the BLE Mesh timeout.

6.  Manual AC Control ACKs (ID:4 and ID:104) are never triggered by cloud commands. They fire automatically whenever the IR receiver detects a physical remote press.

7.  The locking feature: if Locking=1 and the user presses the physical remote with a temperature outside TempLockLowLimit–TempLockUpLimit, the firmware overrides the AC back to the boundary temperature and the corrected state is reflected in currACState.