BLE Mesh Group Control

MQTT Packet Reference & User Guide

Unimation   |   Tue Sep 01 2026

1. Overview

Group control allows the cloud to manage multiple AC units simultaneously using a single command. Instead of sending individual commands to each node, the cloud assigns nodes (and optionally the gateway) to a group address and then sends one command to that address.

There are three key operations:

•  Subscribe — add a device (gateway or node) to a group address

•  Unsubscribe — remove a device from a group address

•  Group AC Control — send an AC control command to all devices in a group

ℹ  GroupAddr must be between 49152 (0xC000) and 65534 (0xFFFE). MQTT topic for commands: {GwySerNo}/command. MQTT topic for responses: {GwySerNo}/message.

2. How Group Control Works

2.1 Group Address

A group address is a shared BLE Mesh address that multiple devices can listen on. When the gateway publishes a message to a group address, all devices subscribed to that group receive and process it simultaneously — a single BLE Mesh transmission reaches all members at once.

2.2 Two Types of Members

| Member Type | ElementAddr | Description |
| --- | --- | --- |
| Gateway (GWY) | 1 | The gateway itself controls its own locally-connected AC via IR |
| Node (NODE) | 5, 7, 9... | A BLE Mesh node that controls an AC via IR at its location |

2.3 Subscribe vs Unsubscribe

Subscribing a device to a group tells its BLE Mesh stack to listen on that group address. Unsubscribing removes it. The gateway maintains a group table in NVS flash that persists across reboots — so group memberships survive power cycles.

2.4 Group AC Control — Two ACK Responses

When a group AC control command is sent, the gateway sends back two separate ACK packets:

•  Immediate ACK (ID:119) — sent instantly after the gateway dispatches the BLE Mesh group message. Tells the cloud how many nodes the command was sent to.

•  Summary ACK (ID:120) — sent after all nodes confirm execution OR after a 10 second timeout. Shows exactly which nodes succeeded and which failed.

ℹ  If the gateway itself is subscribed to the group, it executes the IR command locally before sending the BLE Mesh group message — so the gateway AC is controlled in the same operation.

3. Subscribe to a Group

3.1 Subscribe Gateway to Group — Cloud → Gateway (ID:14)

Send this packet to subscribe the gateway itself to a group. Use ElementAddr: 1 (always 1 for the gateway). After this, group AC control commands to this group will also control the gateway's own AC.

Request — Cloud publishes to {GwySerNo}/command:

{

"JsonPacketID": 14,

"MsgSeqNo": 1,

"ElementAddr": 1,

"GroupAddr": 49153

}

Response — Gateway publishes to {GwySerNo}/message (ID:16):

{

"JsonPacketID": 16,

"MsgSeqNo": 1,

"GwySerNo": "GWY00013",

"ElementAddr": 1,

"GroupAddr": 49153,

"ErrorCode": 0,

"ErrorMsg": "SUCCESS"

}

3.2 Subscribe Node to Group — Cloud → Gateway → Node (ID:114)

Send this packet to subscribe a specific BLE Mesh node to a group. The gateway forwards this to the node over BLE Mesh. The node subscribes its own model to the group address and confirms back to the gateway which then reports to the cloud.

Request — Cloud publishes to {GwySerNo}/command:

{

"JsonPacketID": 114,

"MsgSeqNo": 3,

"ElementAddr": 5,

"GroupAddr": 49153

}

Response — Gateway publishes to {GwySerNo}/message (ID:16):

{

"JsonPacketID": 16,

"MsgSeqNo": 3,

"GwySerNo": "GWY00013",

"NodeSerNo": "N00010",

"ElementAddr": 5,

"GroupAddr": 49153,

"RSSI_dBm": -35,

"ErrorCode": 0,

"ErrorMsg": "SUCCESS",

"BleErrorCode": 0

}

ℹ  Both GWY subscribe (ID:14) and NODE subscribe (ID:114) return the same ACK packet ID:16. Use ElementAddr to distinguish — ElementAddr:1 means gateway, any other value means a node.

4. Unsubscribe from a Group

4.1 Unsubscribe Gateway from Group — Cloud → Gateway (ID:15)

Removes the gateway from a group. After this, group AC control commands to this group will no longer affect the gateway's AC.

Request:

{

"JsonPacketID": 15,

"MsgSeqNo": 2,

"ElementAddr": 1,

"GroupAddr": 49153

}

Response (ID:17):

{

"JsonPacketID": 17,

"MsgSeqNo": 2,

"GwySerNo": "GWY00013",

"ElementAddr": 1,

"GroupAddr": 49153,

"ErrorCode": 0,

"ErrorMsg": "SUCCESS"

}

4.2 Unsubscribe Node from Group — Cloud → Gateway → Node (ID:115)

Removes a specific node from a group. The gateway forwards this to the node over BLE Mesh. The node unsubscribes its model from the group address.

Request:

{

"JsonPacketID": 115,

"MsgSeqNo": 4,

"ElementAddr": 5,

"GroupAddr": 49153

}

ℹ  Unsubscribe ACK uses ID:17 for both gateway and node unsubscribe, same as subscribe uses ID:16 for both.

5. Group AC Control

5.1 Group AC Control Command — Cloud → Gateway → All Members (ID:118)

Sends an AC control command to all devices subscribed to the specified group address simultaneously. The gateway checks its group table — if it is subscribed, it controls its own AC first, then broadcasts the BLE Mesh group message to all subscribed nodes.

Request — Cloud publishes to {GwySerNo}/command:

{

"JsonPacketID": 118,

"MsgSeqNo": 5,

"GroupAddr": 49153,

"Power": 1,

"Mode": "Cool",

"FanSpeed": 3,

"Temperature": 24,

"SwingH": 0,

"SwingV": 0,

"OnTimer": 0,

"OffTimer": 0,

"Locking": 0,

"TempLockUpLimit": 32,

"TempLockLowLimit": 18

}

Response 1 — Immediate ACK (ID:119) — sent within milliseconds:

{

"JsonPacketID": 119,

"MsgSeqNo": 5,

"GwySerNo": "GWY00013",

"GroupAddr": 49153,

"GwySubscribed": 1,

"NodeCount": 3,

"ErrorCode": 0,

"ErrorMsg": "SUCCESS"

}

| Field | Type | Description |
| --- | --- | --- |
| GwySubscribed | int | 1 if gateway was in this group and executed locally, 0 if not |
| NodeCount | int | Number of BLE Mesh nodes the command was sent to (excludes gateway) |

Response 2 — Summary ACK (ID:120) — sent after all nodes respond or 10s timeout:

Success case — all nodes confirmed:

{

"JsonPacketID": 120,

"GwySerNo": "GWY00013",

"GroupAddr": 49153,

"Seq": 1,

"GwySubscribed": 1,

"ExpectedNodes": 3,

"ReceivedAcks": 3,

"FailedCount": 0,

"FailedNodes": []

}

Partial failure case — one node did not respond:

{

"JsonPacketID": 120,

"GwySerNo": "GWY00013",

"GroupAddr": 49153,

"Seq": 1,

"GwySubscribed": 1,

"ExpectedNodes": 3,

"ReceivedAcks": 2,

"FailedCount": 1,

"FailedNodes": [9]

}

| Field | Type | Description |
| --- | --- | --- |
| Seq | int | Sequence number matching this command — use to correlate with Immediate ACK |
| GwySubscribed | int | 1 if gateway was in group and executed locally |
| ExpectedNodes | int | Total node count the command was sent to |
| ReceivedAcks | int | Number of nodes that confirmed execution |
| FailedCount | int | Number of nodes that failed or did not respond within 10 seconds |
| FailedNodes | array | ElementAddr values of nodes that failed — empty array if all succeeded |

ℹ  Multiple consecutive group commands are tracked independently using the Seq number. Each command gets its own Summary ACK even if another command was sent before the first one completed.

6. Query Current Group Table

Send this packet to get the full list of all group subscriptions currently stored on the gateway. The gateway also publishes this automatically every time it reconnects to MQTT so the cloud can sync its local state.

Request (ID:18) — Cloud publishes to {GwySerNo}/command:

{

"JsonPacketID": 18,

"MsgSeqNo": 1

}

Response (ID:18) — Gateway publishes to {GwySerNo}/message:

{

"JsonPacketID": 18,

"MsgSeqNo": 1,

"GwySerNo": "GWY00013",

"GroupCount": 2,

"Groups": [

{

"GroupAddr": 49153,

"MemberCount": 3,

"Members": [

{"ElementAddr": 1, "Type": "GWY"},

{"ElementAddr": 5, "Type": "NODE"},

{"ElementAddr": 7, "Type": "NODE"}

]

}

]

}

ℹ  Use this after a gateway reconnect or on cloud startup to sync your local group table with the gateway's persisted state.

7. Packet ID Summary

| ID | Name | Direction | Description |
| --- | --- | --- | --- |
| 14 | GWY_GROUP_SUB_PACKET | Cloud → Gateway | Subscribe gateway to group |
| 15 | GWY_GROUP_UNSUB_PACKET | Cloud → Gateway | Unsubscribe gateway from group |
| 16 | GWY_GROUP_SUB_ACK | Gateway → Cloud | Subscribe confirmation (GWY or NODE) |
| 17 | GWY_GROUP_UNSUB_ACK | Gateway → Cloud | Unsubscribe confirmation (GWY or NODE) |
| 18 | GWY_GROUP_TABLE_PACKET | Both directions | Query group table / group table response |
| 114 | NODE_GROUP_SUB_PACKET | Cloud → Gateway → Node | Subscribe node to group |
| 115 | NODE_GROUP_UNSUB_PACKET | Cloud → Gateway → Node | Unsubscribe node from group |
| 118 | NODE_GROUP_AC_CONTROL_PACKET | Cloud → Gateway → Group | AC control to all group members |
| 119 | NODE_GROUP_AC_CONTROL_IMMEDIATE_ACK | Gateway → Cloud | Immediate dispatch confirmation |
| 120 | NODE_GROUP_AC_CONTROL_SUMMARY_ACK | Gateway → Cloud | Final result after all nodes respond |

8. Typical Setup Workflow

Follow this sequence to set up group control for a zone of AC units:

| Step | Action | Packet |
| --- | --- | --- |
| 1 | Subscribe the gateway to group 0xC001 (if gateway AC is part of the zone) | ID:14 with ElementAddr:1 |
| 2 | Subscribe each node to group 0xC001 one by one | ID:114 with each node's ElementAddr |
| 3 | Wait for all subscribe ACKs (ID:16) to confirm membership | ID:16 response |
| 4 | Send group AC control command to 0xC001 | ID:118 with GroupAddr:49153 |
| 5 | Receive immediate ACK confirming command dispatched | ID:119 |
| 6 | Receive summary ACK confirming all nodes executed | ID:120 |
| 7 | Check FailedNodes — retry individual unicast for any failed nodes if needed | ID:103 per failed node |

ℹ  GroupAddr 49153 = 0xC001 in decimal. All GroupAddr values in MQTT packets use decimal integers.